| Name | Ashutosh Thanvi |
| --- | --- |
| DTU ID | S252568 |
| Course | Scientific Programming in Wind Energy |
| Assignment 3 | Wind turbine modelling |
| Date | 05th December 2025 |

Executive Summary

A wind turbine modelling tool was developed in python using Blade Element Momentum (BEM) theory. The objective of this project is to develop a steady-state Blade Element Momentum (BEM) model to predict the aerodynamic performance of a wind turbine. The model will compute key performance metrics such as power output, thrust, and torque as functions of wind speed, rotor speed, and blade pitch angle.

Algorithm chart flow

The below figure shows the algorithm of the tool.

Interpolate for omega and pitch for every wind speed based on operational strategy

Plot Power and Thrust as a function of wind speeds

Plot aerofoil shapes

Application of Blade Element Momentum theory include loss corrections

Extract Aerofoil data & Operational conditions

Read Blade Geometry and Operational conditions

Blade Geometry - Chord, twist, air foil data along the span

Operational conditions – wind speed , rotor speed, pitch angle

Air foil data for each aerofoil - x/c, y/c, Cl and Cd as a function of angle of attack

### Methodology- BEM

Consider a wind turbine rotor with radius R, we can consider a blade element at span position r with element length of dr, as shown in below:

Source: Figure 3.8 in (Hansen, 2015)

We can then calculate the local solidity σ(r), defined as the fraction of the annular area in the control volume, which is covered by blades as follows:

σ(r)=c(r)B2πr

where B denotes the number of blades that is 3 for the turbine we considered.

Assume the local chord length at this span position is c(r), the local twist angle is β(r), and the rotor is operated at the following conditions:

- Inflow wind speed: V0

- Blade pitch angle: θp

- Rotational speed: ω

Note that given the rotor radius R, the inflow wind speed V0 and the rotational speed ω, the tip speed ratio λ, or TSR, is given by:

λ=ωRV0

Based on the procedure described in Chapter 6 of (Hansen 2015), the following steps can be taken to solve the steady BEM equations to obtain the axial (a) and tangential (a') induction factors and compute the contribution of thrust and torque of this blade element:

- Step 1: Initialize a and a', typically a=a'=0.

- Step 2: Compute the flow angle ϕ using the following equation:

tan⁡ϕ=(1-a)V0(1+a')ωr

- Step 3: Compute the local angle of attack α using the following equation:

α=ϕ-(θp+β)

- Step 4: Compute Cl(α) and Cd(α) by interpolation based on the airfoil polars.

- Step 5: Compute Cn and Ct with:

Cn=Clcos⁡ϕ+Cdsin⁡ϕ

Ct=Clsin⁡ϕ-Cdcos⁡ϕ

- Step 6: Update a and a' with:

a=14sin2⁡ϕ/[σ(r)Cn]+1

a'=14sin⁡ϕcos⁡ϕ/[σ(r)Ct]-1

- Step 7: If a and a' have changed more than a certain tolerance, go back to Step 2, otherwise continue to Step 8.

- Step 8: Compute the local contribution of this blade element to the thrust and torque as:

dT=4πrρV02a(1-a)dr

dM=4πr3ρV0ωa'(1-a)dr

Loop over all blade elements, we can then integrate to get the thrust (T) and torque (M), then the (aerodynamic) power output of the rotor can be computed as:

P=Mω

Results

Aerofoil shapes for 50 aerofoils along the blade span

Rotor Power (MW) Vs Wind speed (m/s) and Thrust (kN) Vs Wind speed (m/s)

The rates power of the turbine is 15MW and the rated speed is 11m/s. The thrust increases up to rated wind speed and then decreases due to pitching of blades.